CSST

McIntyre laboratory Safety Inspection Form

## Interpretation Criteria

| # GENERAL LAB ORGANIZATION |  |
| --- | --- |
| 1. Work surfaces clean |  |
| 2. Work surfaces in good condition | Mark "x" for cracked, pitted, peeling, damaged or soiled surfaces |
| 3. Work surfaces decontaminated. Frequency: | Biohazardous and radioactive work surface decontamination required after each experiment. For other experimental work, see inspection item #1. |
| 4. Ease of pedestrian circulation | Floors should be clear of tripping hazards and any obstacles blocking emergency equipment or exits. |
| 5. Floor clean, not slippery | Mark "x" for wet or visibly dirty floors |
| 6. Walls and ceilings in good condition | Report any observed structural defects, note the location of stained ceiling tiles and advise Facilities Management of the situation. |
| 7. Chairs & stools with impermeable covers | Stools and chairs used for experimental work must be covered with impermeable material. Upholstered chairs are acceptable if used exclusively at study desks |
| 8. No large or heavy objects stored above1.25 m | Large objects that present ergonomic risks (lifting too high) or safety risks (falling upon people or experimental areas) should be stored below 1.25 m |
| 9. Shelves level and affixed to wall | Self-explanatory |
| 10. Objects do not overhang shelves | Self-explanatory. A recommended solution to prevent this is the installation of shelf edge-guards |
| 11. Clear access to doors | No tripping hazards or obstacles in the way of doors |
| 12. Lab doors closed, not locked | Further, if the lab is unattended, the doors must be closed AND locked |
| 13. Signage attached to wall, away from heat | Not a common problem |
| 14. Disinfectants available. Specify: | Biohazard policy specifies that disinfectants be readily available |
| ADMINISTRATION |  |
| 15. Lab work separated from office work | Desks in labs are acceptable, but not to be used for experimental work |
| 16. Procedures available and updated annually | Ref: CSST entente |
| 17. SOP's initialled | Not a McGill requirement |
| 18. H&S policies and procedures available | Lab personnel should be aware of policies/procedures. See www.mcgill.ca/eso |
| 19. H&S Officer for lab | Not a McGill requirement. The lab director is responsible for the lab. Appointment of safety officer optional. Department Safety Committees |
| 20. Registry of personnel training up-to-date | ESO has lists of persons trained in Radiation Safety, WHMIS, etc. |
| 21. Emergency equipment maintenance records | Not applicable in individual labs - Emergency equipment is managed by other departments (WMP, Security, etc.) |
| # PHYSICAL CONDITIONS |  |
| 22. Room ventilation operational | Fume hood and air supply diffusers should be in operation. |
| 23. Lab under negative pressure | Verified by Facilities Management (not by lab inspectors) |
| 24. Registry of negative pressure checks | Verified in Facilities Management balancing report (not by lab inspectors) |
| 25. Noise level permits normal conversation | Not a common problem |
| 26. Noisy equipment in separate room | Not a common problem |
| 27. Comfortable room temperature | Self-explanatory |
| 28. Comfortable humidity level | Self-explanatory |
| 29. Sufficient room lighting | Self-explanatory |
| 30. Task lighting available | Optional - Room lighting is sufficient for most tasks |
| HYGIENE |  |
| 31. Hand soap near sinks | Self-explanatory. Waterless cleaners are acceptable if there's no sink |
| 32. Paper towels near sinks | Self-explanatory. Waterless cleaner acceptable, if there's no sink |
| 33. Hand cream available | Hand creams not required |
| 34. Latex-compatible hand cream | Hand creams not required |
| 35. Proper hand-washing techniques | Self-explanatory. Waterless cleaner acceptable, if there's no sink |
| 36. Storage of personal effects | Personal items not to be stored in experimental areas |
| 37. No food and drink | No food & drink permitted in the lab. |
| 38. Long hair restrained | Long hair must be restrained when working with flames, machinery or hazardous materials |
| 39. Fingernails short, with clear or no polish | Not applicable |
| 40. Closed shoes | No sandals or open heeled shoes (such as clogs). |
| WHMIS |  |
| 41. MSDS accessible | MSDS should be readily available. MSDS location should be indicated on Lab Card. |
| 42. MSDS up-to-date (less than 3 yrs) | Date of preparation of MSDS must be within 3 years. |
| 43. WHMIS posters visible | Not a McGill requirement |
| 44. Containers labelled | As per WHMIS requirement |
| 45. Legible labels | Self-explanatory |
| 46. WHMIS core training | Did personnel attend an ESO or department WHMIS seminar? |
| 47. WHMIS specific training | Did lab director or supervisor provide safety info specific to the labs? |
| 48. Personnel aware of hazards of products | Big question! Ask personnel if they're aware of the safety manuals and MSDS collection. |
| BIOLOGICAL PRODUCTS AND SPECIMENS |  |
| 49. Visible biohazard signs and labels | Check Lab Information Card |
| 50. Containment level posted. Level: _____ | Check Lab Information Card |
| 51. Specimens handled according to risk group | Verified by biohazard certification process |
| 52. Specimens stored according to risk group | Must be properly identified, secured from public |
| 53. Aerosols adequately contained | Aerosol producing experiments should be performed in biological safety cabinet |
| 54. Sealed safety cups for centrifuge | Required if biohazardous samples are centrifuged |
| 55. Ultracentrifuge rotor inspected yearly | Check log or ask user. |
| 56. Safe transport of specimens | If they transport, do they follow TDG regulations? |
| 57. Proper disposal of waste | Check their waste containers and see if they are properly labelled |
| 58. Sharps disposal container | Labelled, puncture-proof container required |
| 59. Apparatus for picking up broken glass | Dust pan and whisk broom recommended |
| CHEMICAL PRODUCTS |  |
| 60. Yearly inventory available | Should be in transparent folder on bulletin board |
| 61. Flammable liquid containers exceeding 1 litre | See table of maximum container sizes in Lab Safety Manual. Justify why containers > 1 litre are needed. |
| 62. Corrosives containers exceeding 1 litre | Justify why containers > 1 litre are needed. |
| 63: Use of ether | Can ether be eliminated? |
| 64. Flammable liquid storage cabinet | Flammable liquids shouldn't be stored out in the open |
| 65. Chemical segregation | Flammable solvents stored away from oxidizers; acids, & bases stored separately. Refer to Lab Safety Manual segregation sections. |
| 66. Flammable storage in appropriate fridge | Not recommended. Store flammables under hood or in flammable liquid storage cabinet |
| 67. Volatile chemicals handled in hood | Self-explanatory |
| 68. Expiry dates on unstable chemicals | Look for ether, dioxane, tetrahydrofuran |
| 69. No expired unstable chemicals | Look for ether, dioxane, tetrahydrofuran |
| 70. Condition of containers inspected | Look for signs of deteriorating labels or damaged chemical containers. Monthly |
| 71. Hazardous chemicals stored below 1 m height | To minimize splash hazards |
| 72. Spill trays or basins under corrosives | Pyrex or polypropylene trays recommended |
| 73. Containers firmly closed | Self-explanatory |
| 74. Containers protected from heat, direct sunlight | No chemicals on window sills or near heaters, furnaces, hot plates, etc. |
| 75. Flammables away from ignition sources | No flammables near heaters, furnaces, ovens, flames, hot plates, etc. |
| 76. Toxic products stored in secure location | Controlled drugs, euthanasia agents, cyanides etc. stored under lock and key |
| # COMPRESSED GASES |  |
| 77. Type(s) present: | List the types present: |
| 78. WHMIS label legible | Self-explanatory |
| 79. Cylinders secured | Chained (preferable) or belted to bench or wall |
| 80. Restraints in good condition | Check chain or belt |
| 81. Regulator in good condition | Any signs of corrosion or damage to regulator? |
| 82. 100 psi residual pressure in empty cylinders | Not possible for inspector to verify |
| 83. Empty cylinders identified | "MT" legible on cylinder |
| PERSONAL PROTECTIVE EQUIPMENT (PPE) |  |
| 84. Lab coats | To be worn while conducting experimental work |
| 85. Gowns | Can be worn instead of a lab coat |
| 86. Latex gloves | Can be worn for work with biological fluids, not recommended for chemicals |
| 87. Nitrile gloves | Suitable for work with many types of chemicals, biological materials and radioisotopes |
| 88. Thin cotton gloves | Not a safety requirement. Some individuals with sensitive skin may use these as glove liners |
| 89. Insulated gloves | For manipulations of hot items. |
| 90. Cut-resistant gloves | Self-explanatory |
| 91. Anti-vibration gloves | Not required for routine lab work |
| 92. Other gloves (specify): | List the types presents: |
| 93. Anti-impact, shatterproof safety glasses | To be worn whenever conducting or in close proximity to experimental work |
| 94. UV, IR, laser proof glasses | For work with open lasers or UV environments |
| 95. Shoe covers | Not required for routine lab work |
| 96. Aprons | Can be worn instead of a lab coat |
| 97. Hearing protection | Not required for routine lab work |
| 98. Mask (specify type): | Not required for routine lab work |
| 99. Training for mask | Not required for routine lab work |
| 100. Respirator (specify type): | Not required for routine lab work |
| 101. Respirator training | Not required for routine lab work |
| 102. Respirator fit-test (specify type): | Required for personnel who perform HAZMAT functions |
| 103. Other PPE (specify): | List the types present: |
| 104. PPE training | Required for specialized applications only e.g. HAZMAT teams |
| 105. PPE available in sufficient quantity | Self-explanatory |
| 106. PPE available in sufficient sizes | e.g. a variety of glove sizes |
| 107. PPE inspected before use | Not required for disposable i.e. single-use PPE |
| 108. PPE adequately maintained | Not required for disposable i.e. single-use PPE |
| 109. Proper disposal of PPE | Most PPE not considered as hazardous waste |
| EMERGENCY MEASURES |  |
| 110. Evacuation routes known to personnel | Route posted in corridor? |
| 111. Evacuation routes obstruction-free | Building director performs weekly checks |
| 112. List of certified first-aiders | Is there a poster in the department? |
| 113. Emergency phone # list near phone | Self-explanatory |
| 114. Biohazard spill protocol available | As per biosafety manual. Only required for users of biological materials |
| 115. Biohazard contamination warning sign | Only needed for spills, can be improvised, WMP has "danger" tape |
| 116. Biohazard spill materials available | As per biosafety manual. Required for users of biological materials |
| 117. Chemical spill protocol available | Emergency phone # list is sufficient |
| 118. "Danger, do not enter" signs or tape available | Available from Waste Management Program, room 129 |
| 119. Chemical spill kit available | Waste Management Provides spill response assistance. Spill kit in each lab is recommended, but not mandatory |
| 120. Emergency shower available | Should be within 10 seconds walking distance of experimental area. (Where corrosive chemicals are used?) |
| 121. Emergency eyewash available | Should be within 10 seconds walking distance of experimental area. (Where corrosive chemicals are used?) |
| 122. Fire extinguisher | Should be 1 in each lab. Verify inspection tag is up to date and that the seal is intact. |
| 123. Training in the use of emergency equipment | Fire extinguisher training is the responsibility of the Fire Prevention Office |
| 124. Emergency drills. Frequency: | 1/Year. Conducted by Fire Prevention Office |
| ELECTRICAL SYSTEMS |  |
| 125. No overloaded circuits | Not for lab inspector to verify. Report suspected problems to Facilities Management |
| 126. Grounding | Not for lab inspector to verify. Report suspected problems to Facilities Management |
| 127. Breakers in place and functional | Not for lab inspector to verify. Report suspected problems to Facilities Management |
| 128. Fuses | Not for lab inspector to verify. Report suspected problems to Facilities Management |
| 129. Cables, wiring, outlets in good condition | Not for lab inspector to verify. Report suspected problems to Facilities Management |
| 130. No wires under doors, carpets, on ceiling | Self-explanatory |
| 131. Overloaded or excessively long extensions | Self-explanatory |
| 132. Equipment adequately ventilated | Not for lab inspector to verify. Report suspected problems to Facilities Management |
| 133. High voltage warning labels | On high voltage power supplies, usually applied by the manufacturer. |
| CHEMICAL FUME HOODS |  |
| 134. Hood available | Self-explanatory |
| 135. Face velocity verified yearly | Performed by Facilities Management |
| 136. Uncluttered | A judgement call - remove items not being used on an ongoing basis. Raise large equipment upon blocks |
| 137. No storage of products in hood | Maximum 1-2 day supply of chemicals |
| 138. No storage of wastes in hood | Self-explanatory |
| 139. Velocity indicator, not muted | Verify that velocity indicator is functional and that the alarm is not muted. |
| BIOLOGICAL SAFETY CABINETS (BSC) | Omit this section if biological materials are not used |
| 140. BSC available | BSC = biological safety cabinet |
| 141. Certified annually | Verify that BSC has a valid certificate |
| 142. Work surface and shield clean | Self-explanatory |
| 143. Stored materials (specify): | Only materials used for an ongoing experiment are permitted inside a BSC |
| 144. Permanent apparatus (specify): | Only materials used for an ongoing experiment are permitted inside a BSC |
| 145. No Bunsen burners in BSC | Micro incinerators or burners with push activated flames are recommended |
| 146. Corrosives or solvents used in BSC | Only material used for an ongoing experiment are permitted inside a BSC |
| 147. Front and rear grills unobstructed | Self-explanatory |
| 148. Lighting functional | Self-explanatory |
| 149. Air flow indicator | Self-explanatory |
| 150. Personnel aware of safe use of BSC | Have they attended one of the annual seminars been trained by their supervisor, or used EH&S instructional material? |
| Additional Comments: |  |

/tmp/in/input.doc